LOOK UP!

Bryan Wagner

Overhead hazards are a clear and present danger in our forest industry.  Our industry on a daily basis deals with height, weight and gravity issues.  When something goes wrong, these properties are proven killers to our woods workers.  Catastrophic injury and death are the results of overhead hazards.  We must fight the risk from above by being alert and aware.  Safety briefings by management can help keep awareness at a good level.  Overhead hazards are a “team” battle.  Our logging crews or teams can reduce the risk of overhead hazards by working together.  Overhead hazards must first be spotted or identified.  When a hazard is found, proper decisions and actions can defuse the risk.  The process starts by LOOKING UP!

- Proper, well maintained head protection (hard hats) are required and mandatory on a logging job.  This includes anyone on the ground, outside the protection of equipment that is equipped with ROPS and FOPS.  Equipment operators are encouraged to wear head protection even while in the safety of an enclosed cab.  Hard hats can prevent injury in the case of machine roll over, or from sudden, unplanned stops.

- Look Up and Out!  Scan your ground travel path at least 50 ahead of you.  Scan at least 100 feet ahead for overhead hazards.

- Don’t place yourself under anything that can move due to gravity or the loss of hydraulic pressure.

- Maintain at least two tree lengths from any felling area.  The industry standard for the two-tree length rule is 300 feet, or 100 yards.  This distance can be highlighted in safety meetings, one hundred yards is a common rifle sight-in range.  One hundred yards is the length of a football field.

- Be Visible!  Be Seen!  Make yourself visible by wearing high contrast clothing and high visibility safety colors.

- Never walk into a work area with running and moving equipment.  Equipment must be stopped, engine cut-off, and all hydraulic implements grounded.

- Use proper felling techniques.  Directional felling techniques reduce the risk of “struck-by” accidents.  Proper notching and adequate hinge wood insures a timber cutter’s safety.

Take the time to review the risk of overhead hazards with your employees, or team.  Heightened awareness reduces the risk of injury or death to our woods workers and loved ones.  Remember, Look Up and Live!